bGeigieZen NFW: Simplified Assembly Guide

Safety Warnings

- Work in a well-ventilated area (solder smoke is hazardous)

- Wear safety glasses when cutting leads

- Soldering iron temperature: 200-350°C (400-650°F)

- Handle components gently, especially the LND_7317 pancake sensor (mica covering is easily damaged)

- NEVER short circuit an 18650 battery (fire/explosion hazard)

Required Tools/Parts

- Soldering iron with fine tip and solder (60/40)

- Nippers for cutting pin headers

- Screwdrivers (small flat-head and Phillips)

- USB-C cable for programming/charging

- Optional: Isopropyl alcohol for cleaning flux

- bGeigieZen NFW kit parts are below

Extra Components

- Battery: 650 mm 3.7V 3000mA unprotected flat-top type

Assembly Steps

1. Prepare the Board

- Solder the 2×15 pin (#8 in parts picture) connector to the board

- Place the pin connector with top up on the board(#5 in parts picture) , turn over the board while holding the pin header

- Solder one corner pin, check alignment, then solder remaining pins

- Solder the 1×3 pin header(#7 in parts picture) for the Safepulse (#13 in parts picture)

- Solder the 1×2 pin header(#7 in parts picture) for the Safepulse(#13 in parts picture)

2. Install Diode, Fuse, and Wireless Charger Connector

- Place the diode(#17 in parts picture) with the white band pointed to the fuse (#15 in parts picture)

- Check orientation and placement carefully

- Solder the diode, fuse, and the wireless charger connector in place

- Cut the extra length of the leads of the diode off.

3. Install the Safepulse Module

- Position the board with the backside up

- Solder all pins of the 3 pins header and the 2 pin header on the board

- place the safepulse and solder the pins

- Solder the wire for the anode connector (red) (#11 in parts picture)

4. Prepare and Install the GPS Module

- For M10050-HT2828 module, follow the connector pins picture below

| GPS module | bGeigieZen board |
| --- | --- |
| G | G |
| R | TX |
| T | RX |
| V | 5V |

5. Install Battery Holders

- Install battery holder clips in appropriate holes

- Solder one pin of each holder, check alignment, then solder remaining pins

- Ensure flat bottom of the holder touches the board

6. Prepare and Install the Tube

- HANDLE WITH EXTREME CARE - grid is fragile

- Place tube with grid down into case in the cutout hole of the rubber

- remove the 3M stick paper of the tube

- Place board in case without moving tube

- Install M3 15mm screws for mounting M5StackCore (DON'T OVERTIGHTEN)

- Unroll the cathode (blank wire connected to the tube) and place the shrink tube (#10 in parts picture) over the blank wire

- Solder the blank wire to the Safepulse (labled as cathode).

- Connect the anode connector (red wire) to the tube and add protected shrink tube (#9 in parts picture). Heat the shirnk tube to about 120 degree Celcius

- The assambly should now look like(without battery):

- Insert SD card

7. Final Assembly

- For QI charging:

- Cut hole for receiver cable

- Solder QI receiver (green small board with LEDs) red to plus and black to minus

- Close case (should have soft "rubber touching case" sound)

- Confirm that top of rubber liner is 2mm above the board

Software Installation

- Install USB VCP Driver

- Install M5Burner/UIFlow Firmware Burning Tool

- Connect the M5StackCore via USB-C

- Select your M5StackCore type and search for "Zen"

- Download and burn the firmware

- Configure by editing the SAFEZEN.txt file on SD card

Notes and Tips

- When powering down, wait a few seconds before powering up again

- Ensure the aluminum connector of the anode is firmly screwed on (loose connections cause incorrect counts)

- Most kits already have protective mesh screen on sensor (check before assembly)

For detailed instructions or troubleshooting, refer to the complete manual at Link